AIRPORTS

INTRODUCTION TO AIRPORTS

AIR TRAFFIC BY THE NUMBERS

NATIONAL AIRSPACE SYSTEM

- Network of airspace, navigation facilities, airports, technology, and regulations that enable safe and efficient air travel in the US.

- Airspace Management - Organizes airspace: controlled, uncontrolled & special use

- Communication Systems- between pilots and controllers.

NATIONAL AIRSPACE SYSTEM

- Navigation & Surveillance: Uses radar, GPS, and other systems to track aircraft and provide navigation support.

- Air Traffic Services: Includes flight planning, weather services, and traffic flow management.

- Safety & Regulation

AIR TRAFFIC CONTROL

- Primary purpose of the ATC - to prevent a collision between aircraft operating in the system and to organize and expedite the flow of traffic.

  - Provides instructions for flight paths, altitude changes, and landing/takeoff clearances.

  - Separation of Aircraft

  - Traffic Flow Management

  - Emergency Assistance

  - Coordination Across Sectors

LATITUDE AND LONGITUDE

LATITUDE AND LONGITUDE

- One of the most reliable and commonly used forms of navigation for pilots and other individuals.

- Individuals can navigate between airports or other locations.

- Any specific point can be determined by the intersection of the two lines (latitude and longitude)

LATITUDE AND LONGITUDE

- The equator is an imaginary circle between the poles of the Earth.

- The equator is considered the start point for lines running North or South.

- Circles parallel to the equator (lines running East and West) are parallels of latitude.

LATITUDE AND LONGITUDE

- Lines of latitude are used to measure degrees of latitude North (N) or South (S) of the equator. (yellow lines in figure)

- The continental US is located between 25°          and 49° N latitude

LATITUDE AND LONGITUDE

- Meridian lines of longitude are from the North Pole to the South Pole and are at right angles to the Equator

- Shown as red lines in the figure

- The “Prime Meridian,” - passes through Greenwich, England which is used as the zero line.

- Measurements are made in degrees East (E) and West (W)     to 180°.

LATITUDE AND LONGITUDE

- The 48 continental states of the US are between 67° and 125° W longitude.

- Chicago: 42° N latitude, 88° W longitude

LATITUDE AND LONGITUDE

- Degrees of latitude and longitude are divided into 60 minutes.

  - Degrees (°), minutes ('), and seconds (").

  - Latitude is listed first, followed by the longitude

- North and East is positive,

- South and West is negative

- If coordinates are shown as a decimal, conversion is required to use with sectional charts

LATITUDE & LONGITUDE CONVERSION

JFK airport coordinates as an example: 40.6399°N, 73.7797°W.

- Use the whole number part of the decimal as the degrees:

- Multiply decimal parts by 60 to convert them to minutes

- Multiply decimal parts of the minutes by 60 to convert them to seconds.

- Coordinates in the degrees, minutes, & seconds format, for JFK:

40 ° and  73 °

.6399 x 60 = 38.39 minutes or 38.39’        .7797 x 60 = 46.78 minutes, or 46.78’

.39 X 60 = 23	                                         .78 x 60 = 47

Latitude: 40°38’23"N	               Longitude: 73°46’47"W

LATITUDE AND LONGITUDE

LATITUDE AND LONGITUDE

LATITUDE AND LONGITUDE

QUESTIONS/COMMENTS
PLEASE POST  IN THE CHAT

AIRPORTS, CHARTS & DATA SOURCES

AIRPORTS AND AIRPORT DATA SOURCES

- If planning on flying your drone within 5 miles of an airport, it is important to review the current data for that airport.

- Date includes information, such as communication frequencies, services available, closed runways, or airport construction.

- Four sources of information are:

  - Aeronautical Charts

  - Chart Supplement U.S. (formerly Airport/Facility Directory)

  - Notices to Airmen (NOTAMs)

  - Automated Terminal Information Service (ATIS)

CHART SUPPLEMENT U.S. (FORMERLY AIRPORT/FACILITY DIRECTORY)

- The Chart Supplement U.S. (formerly Airport/Facility Directory) provides the most comprehensive information on an airport.

- It contains information on airports, heliports, and seaplane bases.

- The Chart Supplement U.S. is updated in books and digitally and are revised every 56 days.

  - www.faa.gov/air_traffic/flight_info/aeronav

- Information provided in a Chart Supplement U.S. and how it may be interpreted, is in the “Legend Sample” located in the Chart Supplement U.S.

EXAMPLE CHART 
SUPPLEMENT US

Info you will need includes:

- Runway info including elevations

- CTAF radio frequency

- Contact information including phone #

- Weather sources

NOTICES TO AIRMEN (NOTAM)

- Time-critical aeronautical information, temporary in nature or not sufficiently known in advance to permit publication in chart supplements or operational publications.

- Receives immediate dissemination by the NOTAM system.

- The NOTAM information could affect your decision to fly.

- Exercise good judgment & common sense  - review the NOTAMs.

SAMPLE NOTAM

!JFK 07/398 JFK OBST CRANE (ASN 2024-AEA-9387-OE) 404002N0734744W (1.8NM NNW JFK) 189FT (171FT AGL) FLAGGED AND LGTD 2507171151-2701151800

- !JFK - NOTAM is for John F. Kennedy International Airport (JFK)

- 07/398 07 - The 398 NOTAM issued in July

- JFK OBST CRANE - an obstruction (OBST)- a crane near JFK.

- (ASN 2024-AEA-9387-OE) - Number assigned by the FAA

    - You will not need to decode NOTAM on the test

SAMPLE NOTAM CONTINUED

!JFK 07/398 JFK OBST CRANE (ASN 2024-AEA-9387-OE) 404002N0734744W (1.8NM NNW JFK) 189FT (171FT AGL) FLAGGED AND LGTD 2507171151-2701151800

- 404002N0734744W - Latitude/Longitude of crane: 40°40'02" N  73°47'44" W

- (1.8NM NNW JFK) - crane is 1.8 NM N/NW of JFK Airport.

- 189FT (171FT AGL) 189 feet MSL, 171 feet AGL

- FLAGGED AND LGTD - crane is flagged and lighted for visibility.

- 2507171151-2701151800 - Start & end times in UTC

REASONS FOR NOTAMS

- Hazards - air shows, parachute jumps, kite flying, &rocket launches

- Flights - important people - presidents or heads of state

- Inoperable lights on tall obstructions

- Temporary erection of obstacles near airfields

- Flocks of birds through airspace (NOTAM – called BIRDTAM)

- NOTAMs are available at:

  - https://notams.aim.faa.gov/

  - https://tfr.faa.gov/tfr3/?page=list

  - www.1800wxbrief.com

AUTOMATED TERMINAL INFORMATION SERVICE (ATIS)

- The Automated Terminal Information Service (ATIS) is a looped recording of weather conditions and other information broadcast on a local frequency.

  - Weather, runways in use, specific ATC procedures, and any airport construction activity that could affect planning.

- Updated hourly but is updated more often if conditions warrant.

ATIS

- ATIS is given a code and changes with every update. For example, ATIS Alpha is replaced by ATIS Bravo and so on.

- Prior to calling ATC, listen to the recorded broadcast and inform them on initial contact that you have information Bravo or the code given.

- Allows the tower controller to verify the pilot has the current local weather and airport information without having to repeat it

- It is good practice to use the ATIS broadcasts at departure and arrival airports.

AERONAUTICAL CHARTS

AERONAUTICAL CHARTS

- The road map for a pilot.

- The chart provides information that is needed by a remote pilot about the areas where they intend to fly.

- Includes info such as name of the airport, airport class, traffic patterns, radio frequencies, and elevations

- The two aeronautical charts used by drone operators are:

  - Sectional

  - Terminal Area

AERONAUTICAL CHARTS

- Publications used as a learning guide for new pilots or a quick reference guide.

- The FAA is the source for all data and information.

- The data includes planning charts, Visual Flight Rules (VFR) and Instrument Flight Rules (IFR) charts.

- Including training, planning, departures, enroute (for low and high altitudes), approaches, and taxiing charts.

SECTIONAL CHARTS

SECTIONAL CHARTS

- Sectional charts - most common charts used by pilots today

- Named based on major cities on chart

- 1:500,000 scale.

- The charts provide information, including airport data, navigational aids, airspace, and topography.

- Should check the chart for other legend information including air traffic control (ATC) frequencies and information on airspace.

- Charts are revised semiannually

  - Except Alaska  - revised annually

SECTIONAL 
CHARTS

SECTIONAL CHARTS

TERMINAL AREA CHARTS

TERMINAL AREA 
CHART MIAMI

USING AERONAUTICAL CHARTS

- Symbols and terms on the VFR Terms and VFR Sectional & TAC tabs are very useful and include:

  - Airports - locations, runway details, lighting, & services

  - Airspace Information

  - Navigational and Procedural Information

  - Chart Limits

  - Culture - man-made features like cities, roads, railroads, &towers

  - Hydrography – Bodies of Water

  - Relief - Maximum Elevation Figures, contours

SECTIONAL CHART LEGEND

AIRMAN KNOWLEDGE TESTING SUPPLEMENT

USING AERONAUTICAL CHARTS

- Review the symbols in Appendix 1 Legend – used for airports, topographic Information, obstructions, communication boxes and miscellaneous items

- A small flag symbol, which represents a VFR checkpoint

  - Expect higher volume of the air traffic in these areas

AERONAUTICAL CHARTS

- During your planning process determine if your flight will be near an airport.

- Look at the sectional chart for the airport.

- Get authorization if required

AERONAUTICAL CHARTS

Every airport has one on the chart

AIRPORT DATA INCLUDED

- The airport name and location identifier

- The Common Traffic Advisory Frequency (CTAF) or Control Tower (CT) frequency

  - Check airport radio frequencies for incoming & outgoing traffic

- Traffic Pattern – Left or Right

  - Where will air traffic be in relation to you

- Elevation of the runway

  - Where will you be? – determine if risk is acceptable

EXAMPLE AIRPORT DATA

*** Reminder- Blue has a control tower, magenta no control tower

SECTIONAL CHART LEGEND

MAN-MADE OBSTACLES

COMMUNICATION BOXES

Mainly used by aviation pilots but still need to know for the test

MAXIMUM ELEVATION FIGURE (MEF)

QUESTIONS/COMMENTS
PLEASE POST  IN THE CHAT