WEATHER

AVIATION WEATHER SOURCES OVERVIEW

- Weather is an important part of preflight planning

- Many available sources for weather not limited to weather apps, local TV News, and or weather sources

- Weather Services  - collaborative system with multiple agencies including National Weather Service (NWS), Federal Aviation Administration (FAA), Department of Defense (DOD) & other aviation organizations and individuals.

AVIATION WEATHER SOURCES OVERVIEW

- Weather can vary greatly depending on location and altitude

- Aviation weather reports are available at:

  - www.aviationweather.gov

  - 1800wxbrief.com

AVIATION WEATHER SERVICES OVERVIEW

- Weather forecasts are not 100% but are increasingly accurate

  - Use of scientific research & computer models to enhance accuracy.

  - Focus is on identifying patterns, trends, and characteristics of weather systems.

  - Used to support informed decision-making for flight planning and safety.

SURFACE AVIATION WEATHER OBSERVATIONS

- Includes both government-operated and privately contracted facilities.

- Sources will provide real-time, localized weather data critical for aviation operations, collected at ground stations across the US

- Automated weather sources - Automated Weather Observing Systems (AWOS), Automated Surface Observing Systems (ASOS), and other automated facilities play major roles in the gathering of surface observations and data collection

SURFACE AVIATION WEATHER OBSERVATIONS

  - AWOS is a collection of weather sensors of different groupings that were obtained by FAA or purchased by individuals, groups, or airports, that are required to meet FAA standards to be able to report weather factors.

SURFACE WEATHER OBSERVATIONS

- Content of a Surface Observation Report:

  - Type of report -  routine or special

  - Station identifier - airport code or location

  - Date and time of the observation

  - Modifier - if applicable, indicating report updates or corrections

  - Wind speed and direction

  - Visibility - horizontal distance pilots can see

SURFACE WEATHER OBSERVATIONS (CONTINUED)

- Content of a Surface Observation Report Continued

  - Runway Visual Range (RVR) – visibility along the runway

  - Weather phenomena  - e.g., rain, snow, fog

  - Sky condition - cloud cover, ceiling height

  - Temperature and dew point

  - Altimeter reading - atmospheric pressure

  - Remarks – additional relevant information

QUESTIONS

Any questions be sure to leave a comment

AVIATION WEATHER REPORTS

AVIATION WEATHER REPORTS

Purpose:

  - Provide accurate, current depictions of weather conditions for aviation use.

  - Help pilots make informed decisions about flight safety and planning.

Update Frequency:

  - Reports are updated at varying intervals depending on the type and source.

AVIATION WEATHER REPORTS

Common Types of Reports:

  - METAR: Aviation Routine Weather Report - Standard international format for surface weather observations.

    - Issued regularly (typically hourly).

    - Includes - wind, visibility, temperature, sky condition, & more.

  - SPECI: Special METAR - Issued between routine METARs when significant weather changes occur.

  - PIREP: Pilot Weather Report - Provided by pilots in-flight to report actual weather conditions encountered.

AVIATION WEATHER REPORTS
METAR

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

A typical METAR report contains the following info in order:

1. Type of report— two types of METAR reports

  - METAR - report is transmitted on a regular time interval.

  - SPECI - Special report to update the METAR for changing weather conditions, aircraft mishaps, or other critical information.

2. Station identifier – 4 letter code to identify airports & other aviation facilities

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

3. Date & time of report - depicted in a 6-digit group

  - The first two digits are the date.

  - The last four digits are the time of the METAR/SPECI,  always given in coordinated universal time (UTC).

  - “Z”  - time in Zulu time (UTC) not local time.

METAR

Example:  METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

4. Modifier—denotes that the METAR/SPECI came from an automated source or that the report was corrected.

- “AUTO” - report from automated source, might be additional info in the “Remarks” section. Ex: corrected report - replace an earlier report that had an error - METAR KLAX 121852Z COR

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

5. Wind—reported with five digits unless the speed is greater than 99 knots, then reported with six digits.

  - The first 3 digits indicate the direction the true wind is blowing from in tens of degrees.

  - Reported as VRB - If wind is variable

  - Last 2 digits  - speed of wind in knots (3 digits if greater than 99 knots)

  - If the winds are gusting, “G” follows the wind speed (G26KT).

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

- After the letter “G,” the peak gust recorded is provided.

- If the wind direction varies more than 60° & the wind speed is greater than six knots, a separate group of numbers, separated by a “V,” will indicate the extremes of the wind directions.

6. Visibility—reported in statute miles “SM.”

  - Reported in miles and fractions of miles.

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

7. Weather—Broken down into two different categories

- Qualifiers

  - Intensity may be light (–), moderate ( ), or heavy (+).

  - Proximity - only those in the airport vicinity. “VC” – five to ten miles from the airport.

  - Descriptors - describe certain types of precipitation & obscurations

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

- Weather Phenomenon - may be reported as being precipitation, obscurations, and other phenomena, such as squalls or funnel clouds.

METAR

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

8. Sky condition—reported with the amount, height, and type or indefinite ceiling/height (vertical visibility) (BKN008 OVC012CB, VV003).

- Heights of the cloud bases - three-digit number in hundreds of feet AGL

METAR

Ex: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

- VV or vertical visibility is depicted if there is fog or heavy precipitation and clouds can’t be seen

- Types of clouds, specifically towering cumulus (TCU) or cumulonimbus (CB) clouds, are reported with their height.

- Amount of sky coverage is reported in eighths of the sky from horizon to horizon.

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

9. Temperature and dew point- Air temperature & dew point - always given in °C

  - Temps below 0 °C are preceded by the letter “M” to indicate minus.

10. Altimeter setting - inches of mercury ("Hg) in a four-digit number group (A2970).

  - Always preceded by the letter “A.”

  - Rising or falling pressure may also be denoted in the “Remarks” sections as “PRESRR” or “PRESFR,” respectively.

METAR

Example: METAR KLAX 121852Z 25004KT 6SM BR SCT007 SCT250 16/15 A2991

- 11. Zulu time—places the entire world on one time standard.

- 12. Remarks—Comments might appear but would start with “RMK”.

  - May include wind data, variable visibility, beginning & ending times of particular phenomenon, pressure information, and various other info.  ex:  OCNL LTGICCG.  - occasional lightning in clouds & from cloud to ground.

  - Automated stations - use the remarks section to indicate the equipment needs maintenance.

METAR

QUESTIONS

Any questions be sure to leave a comment

AVIATION WEATHER FORECASTS

AVIATION WEATHER FORECASTS

- Use of Observations:

  - Forecasts are essential for preflight planning.

- Types of Forecast Products Pilots Should Know:

  - TAF (Terminal Aerodrome Forecast)

  - FA (Aviation Area Forecast):

  - SIGMET (Significant Meteorological Information):

  - AIRMET (Airman’s Meteorological Information):

  - FB (Winds and Temperatures Aloft Forecast):

AVIATION FORECASTS

- TAF (Terminal Aerodrome Forecast):

- Forecast for specific airports usually for larger airports.

- Covers expected weather within a 5-mile radius of the airport.

- Valid for a 24 or 30 hour time period & updated 4x a day at 0000Z, 0600Z, 1200Z, and 1800Z.

  - Use the same descriptors and abbreviations as in the METAR report.

  - Beneficial to the remote pilot for flight planning purposes.

AVIATION WEATHER FORECASTS

  - FA (Aviation Area Forecast) - Broad regional forecast.

  - SIGMET (Significant Meteorological Information) -Advisories for severe weather hazards (e.g., thunderstorms, turbulence, volcanic ash).

    - Relevant to all aircraft, especially for safety & preflight planning.

  - AIRMET (Airman’s Meteorological Information) - Advisories for less severe weather conditions (e.g., moderate turbulence, icing).

  - FB (Winds & Temperatures Aloft Forecast) - Provides wind direction, speed, & temperature at various altitudes.

CONVECTIVE SIGNIFICANT METEOROLOGICAL INFORMATION (WST)

- Advisories issued for severe thunderstorms producing surface winds greater than 50 knots, hail at the surface that is ¾ inch or larger in diameter, or the presence of tornadoes.

- Issued to alert pilots about thunderstorms that are heavy or precipitation affects 40% or more of an area that is at least 3,000 square miles in size.

QUESTIONS

Any questions be sure to leave a comment

TERMINAL AERODROME FORECAST (TAF)

TAF

The TAF includes the following information in order:

1. Type of report -  2 types

  - Routine forecast (TAF)

  - Amended forecast (TAF AMD).

2. ICAO station identifier  - 4 digit  - same as used in a METAR.

3. Date and time of origin—time & date (081125Z) of TAF origination is given in the six number code with the first two being the date, the last four being the time. Always in UTC with Z following the time block.

TAF

4. Valid period dates and times— The TAF valid period (0812/0912) follows the date/time of forecast origin group.

  - The first two digits (08) - day of the month for  start of the TAF.

  - The next two digits (12) - starting hour (UTC).

  - 09 is the day of the month for the end of the TAF,

  - The last two digits (12) - ending hour (UTC) of the valid period.

  - A forecast period begins at midnight UTC is annotated as 00.

  - If end time of a period is at midnight UTC, it is annotated as 24.

TAF

5. Forecast wind—the wind direction and speed forecast are coded in a five-digit number group.

6. Forecast visibility

  - Given in statute miles and may be in whole numbers or fractions.

  - If the forecast is greater than six miles, it is coded as “P6SM.”

7. Forecast significant weather—weather phenomena coded in the TAF reports in the same format as the METAR.

- 8. Forecast sky condition—same format as the METAR.

- **Only CB clouds are forecast in this portion of the TAF report as opposed to CBs and towering cumulus in the METAR.

TAF

- 9. Forecast change group—significant weather change forecast to occur during the TAF time period,

  - The expected conditions & time period included in this group.

  - “FM” is used when a rapid & significant change, usually within an hour, is expected.   Or

  - “TEMPO” -  for temporary fluctuations of weather, expected to last less than 1 hour.

- 10. PROB30- percentage that describes the probability of thunderstorms & precipitation occurring in the coming hours.

  - ** Not used for the first 6 hours of the 24-hour forecast.

TAF EXAMPLE

KOKC 051130Z 0512/0618 14008KT 5SM BR BKN030 TEMPO 0513/0516 1 1/2SM BR FM051600 18010KT P6SM SKC BECMG 0522/0524 20013G20KT 4SM SHRA OVC020 PROB40 0600/0606

2SM TSRA OVC008CB BECMG 0606/0608 21015KT P6SM SCT040=

TAF EXAMPLE

TAF EXAMPLE CONTINUED

QUESTIONS

Any questions be sure to leave a comment