AIRSPACE CLASSIFICATIONS

INTRO TO AIRSPACE CLASSIFICATIONS

AIRSPACE CATEGORIES

- 2 Categories -  regulatory and nonregulatory broken into

  - Controlled

  - Uncontrolled

  - Special use

  - Other airspace

- Based on complexity/concentration of aircraft movements, type of the operations, level of safety required, and national and public interest.

AIRSPACE INFORMATION

- It is important to understand the various airspace symbols, and the type of airspace you plan on operating in

AIRSPACE CLASSIFICATIONS

- Controlled airspace - generic term covers the different classifications of airspace

- Defined dimensions within which air traffic control (ATC) service is provided in the airspace classification.

- Be sure to note whether MSL or AGL is used when determining altitude

  - Classes A, B, C, and D use MSL and Class E and G use MSL and AGL

AIRSPACE CLASSIFICATIONS

- Airspace of concern to the remote pilot is:

  - Class A – Starts at 18,000 MSL airspace - covers the entire U.S.

  - Class B –busiest airports

  - Class C –mid sized airports

  - Class D –small city airports

  - Class E (4 types within) –least busy airports and covers entire U.S. at some level

QUESTIONS?

CLASS A AIRSPACE

CLASS A

- The highest level of airspace -18,000 feet mean sea level (MSL) up to and including flight level (FL600) (60,000 Feet)

  - 60,000’ is cut to 600 (the hundredths place)

- Includes airspace over water within 12 NM of the coast.

- All pilots must operate aircraft under instrument flight rules (IFR).

- Primary users - high-altitude commercial flights, & long-haul aircraft

- No markings on charts because its vertical boundaries are known and not charted

CLASS B AIRSPACE

CLASS B AIRSPACE

- From the surface to 10,000 feet mean sea level (MSL)

- Around the busiest airports.

- Consists of a surface area and two or more layers

  - Class B airspace area looks like an upside-down wedding cake

  - Think about how the plane lands or takes off from airport

- Remote pilot must receive authorization from ATC before operating in the Class B airspace

CLASS B EXAMPLE

- Atlanta, GA (ATL) (Hartsfield/Jackson)

- Markings are solid blue lines

- Layer altitudes are shown in blue

- Charted in MSL displayed as a fraction for example 125/SFC, and 125/30

CLASS B EXAMPLE

- Baltimore/Washington
 Intl Thurgood
 Marshall Airport

- Markings are solid blue lines

- Layer altitudes are shown in blue

- Charted in MSL displayed as a fraction for example 100/SFC and 100/15

QUESTIONS?

- Put any questions in the chat. I will be sure to post.

CLASS C AIRSPACE

CLASS C

- Normally, from the surface to 4,000 feet (MSL)

- Have an operational control tower, not as busy as a Class B but still important

- Serviced by radar approach control and have instrument flight rules (IFR) operations

- Area individually tailored and usually consists of a surface area with a 5 nautical mile (NM) radius

- Outer circle - 10 NM radius that extends from 1,200’ to 4,000’ MSL

- Authorization required before operating in Class C airspace

CLASS C EXAMPLE

- Toledo Express (TOL) has an operational control tower - requirement for Class C

- Markings – solid magenta lines

- Layer Altitudes written in magenta.

- Inner Ring – Floor - Surface to ceiling of 4,700 feet MSL

- Outer Ring  - Floor: 2,000 feet MSL to ceiling - 4,700 feet MSL

CLASS C EXAMPLE

- Anchorage, Alaska - PANC

- Inner core: Surface to 4,100’ MSL  within 5 NM

- Outer ring – 1,400’ to 4,100’

- Class E extensions for approach paths

QUESTIONS?

- Put any questions in the chat. I will be sure to post.

CLASS D AIRSPACE

CLASS D

- Normally surface to 2,500’ above the airport elevation MSL

  - Surrounding those airports that have an operational control tower.

- Less busy than a Class C airspace

- Class D airspace is one layer, and is individually tailored

- Some will have arrival extensions or departures for Class D or Class E, and some will have a 5 NM radius around airport

- If the control tower is part time the airspace reverts to Class E or G

- Must receive ATC authorization before operating in Class D airspace

CLASS D

- Cincinnati Municipal/
Lunken Field Airport (KLUK)

- Dashed blue lines markings

- Airspace ceiling indicated by a number in a dashed box, its altitude given in hundreds of feet.

- The outer ring 30 represents 3,000 MSL

- Area marked (-21) indicates up to but not including 2,100’ MSL

CLASS D WITH A CLASS E EXTENSION

- Concord-Padgett Regional Airport (JQF)

- Class D airspace - controlled airspace pilots required to communicate with air traffic control if tower is operational.

- This Class D airspace with Class E extension shown with magenta dashed lines

Sample Test Question - (Refer to FAA-CT-8080-2H, Figure 23, area 3.) What is the floor of the Savannah Class E airspace at the shelf area (outer circle)?

QUESTIONS?

- Put any questions in the chat. I will be sure to post.

CLASS E AND CLASS G AIRSPACE

CLASS E

- Least busy airspace

- Class E airspace is the controlled airspace not classified as Class A, B, C, or D airspace.

- A large amount of the airspace over the US is Class E airspace.

- Provides sufficient airspace for the safe control of aircraft during IFR operations.

CLASS E AIRSPACE

- Extends up to, but not including, 18,000’ MSL (lower limit of Class A).

- All airspace above 60,000’ MSL is Class E airspace.

- In areas where charts do not depict a Class E base, Class E begins at 14,500’  MSL.

- In most areas, the Class E airspace base is 1,200 ft AGL.

- In some areas, Class E airspace base is at the surface or 700 feet AGL.

CLASS E AIRSPACE

- Some Class E airspace begins at MSL altitude, instead of an AGL altitude.

- Federal Airways, shown as blue lines on a sectional chart, are usually found within Class E airspace.

- Do not need ATC authorization to operate in Class E  except for when in Class E2

- Class E airspace acts as a transition zone into other airspaces.

CLASS E EXAMPLE

- Shelby-Cleveland County Regional Airport (KEHO) in Class E airspace

- Beginning at 700 feet above ground level (AGL).

- Shown with faded magenta line (vignette)

CLASS E

- Class E Airspace with the floor 1200’ or greater above surface that abuts Class G

- Represented by a blue vignette. The “faded” side of the vignette represents the typical floor of 1,200 feet AGL.

CLASS E EXAMPLE

- Class E2:  Paulding Northwest Atlanta (PUJ)

- Starts at the surface

- Typically, circular and centered on the airport to contain instrument approach procedures.

- Prior authorization from Air Traffic Control (ATC) is required.

CLASS E2 EXAMPLE

- Monrose Rgnl (MTJ) - Class E2- dashed magenta lines with no intersecting lines indicate Class E airspace starting at the surface.

- Class E Transition Area - Faded magenta vignette around the airport indicates Class E airspace beginning at 700’ AGL.

- Class G  - Uncontrolled airspace exists below the Class E transition area, from the surface up to 700’ AGL outside dashed magenta lines.

CLASS E3

- Extension to Class C Surface Area

- Extends Class E airspace from the surface to provide IFR approach or departure paths

- Has an operating control tower

- Depicted by dashed magenta lines extending from Class C

- No ATC authorization required in E3 but needed in adjacent C airspace

CLASS E3 EXAMPLE

CLASS E4

- Class E4: An extension of a Class D or Class E surface area.

- Differs from an E2 area because it is not centered on the airport but is an extension to another designated surface area, typically to accommodate instrument approach and departure paths.

CLASS E AIRSPACE - E4

- No ATC authorization needed

- Dashed magenta line separates the airspace

- Class D airspace  authorization from ATC is needed

CLASS E

- Class E airspace can change its classification based on certain conditions.

- For instance, in small airports with part-time control towers, the airspace changes from Class D to Class E after the tower closes.

BETHEL ALASKA  (PABE) CLASS E2

- Class E  even with blue dashed line

- It indicates controlled airspace beginning at the surface. - can be Class E or Class D.

- Blue dashed line normally is Class D

- But if no operating control tower, same line may be used for Class E

- Bethel - no control tower, airspace starts at the surface and is controlled - Class E.

CLASS G AIRSPACE

- Class G airspace is uncontrolled airspace.

- Class G is anywhere that is not Class A, B, C, D, or E.

- In the U.S., Class G starts at the surface and goes up to where Class E airspace begins—usually 700 or 1,200 feet AGL.

- Class G airspace is not depicted on the sectional chart

- No prior authorization is required to fly

STEPS TO ANSWER AIRSPACE & CHART QUESTIONS

- Read the question and identify what  map or chart is needed.

- Open the Airman Knowledge Testing Supplement for Sport, Recreational, and Private Pilots & locate the correct figure & area referenced in the question.

- Find the airport or location in the question on that chart.

STEPS TO ANSWER AIRSPACE & CHART QUESTIONS

4. Use the Legend (Page 1 1) to determine the airspace classification and if there is a control tower -Blue = controlled airport (towered), Magenta = non towered airport

5.  Identify altitude references - Class A, B, C, and D airspace use MSL and Class E and G may use MSL or AGL

6. Apply the chart information to the question and select the correct answer.

QUESTIONS?

- Put any questions in the chat. I will be sure to post.

SPECIAL USE AIRSPACE

SPECIAL USE AIRSPACE

- Special use airspace is designated for specific activities within a defined area.

- If not involved in those activities may face restrictions when entering this airspace.

- Multiple types of special use airspace listed in the legend - Airman Knowledge Testing Supplement

- As part of your planning you should confirm if you can fly in the area.

SPECIAL USE AIRSPACE

- Special Use Airspace can include:

  - Restricted areas

  - Prohibited areas

  - Warning areas

  - Military operation areas (MOAs)

  - Alert areas

  - Controlled firing areas (CFAs)

SPECIAL USE AIRSPACE

- Size and shape of the area will vary

- The floor and ceiling vary also

- Information regarding the area are listed in the legend

  - Radio frequencies used

  - Active hours

  - Affected altitudes on your sectional chart

  - Controlling agency’s contact information.

SPECIAL USE

EXAMPLES

RESTRICTED AREAS

- Flight possible with restrictions but not when active.

- Check active hours & affected altitudes on the sectional chart it lists the controlling agency.

- Can contain hazards - artillery firing, aerial gunnery, or guided missiles

- Flying in area without authorization may be extremely hazardous

- Depicted with a hatched blue border and the letter R followed by a number

  - Ex: R – 4806 W  Nellis AFR Range Complex used for Military training and testing

PROHIBITED AREAS

- Aircraft of any kind is prohibited

- Established for national security

- Indicated on the chart with a “P” followed by a number such as “47” and special marking as shown in the legend

- Ex: DOE nuclear weapons facility in Amarillo, TX, Camp David and the National Mall in Washington DC

WARNING AREAS

- Similar to Restricted Areas but U.S. does not have sole jurisdiction over airspace over open waters.

- Legally they can’t stop you from flying into a warning area.

- Contain activity that may be hazardous to nonparticipating aircraft

- Sectional charts depict warning areas with a blue-hatched border & a “W” followed by a number

- Start 3 NM from the U.S. coastline over domestic or international waters.

WARNING AREA

MILITARY OPERATIONS AREAS

MILITARY OPERATIONS AREAS

- Can have lower altitude limits due to activity

- Depicted with magenta markings like Alert Areas

- MOAs are depicted on sectional, VFR terminal area, and en route low altitude charts with a name and are sometimes numbered

  - See Gamecock example

EXAMPLE MOA

ALERT AREAS

- Alert areas are depicted on aeronautical charts with an “A” followed by a number (e.g., A-293)

- Depicted with magenta markings like MOA Areas

- May contain a high volume of pilot training or an unusual type of aerial activity.

- There are no requirements to enter Alert areas but pay attention when flying in alert areas

ALERT AREA EXAMPLE

Alert Area A- 293 West of Daytona Beach Intl

CONTROLLED FIRING AREAS (CFA)

- Contain activities that, if not conducted in a controlled environment, could be hazardous to nonparticipating aircraft.

- The difference between CFAs and other special use airspace is that activities must be suspended when a spotter aircraft, radar, or ground lookout position indicates an aircraft might be approaching the area.

- Not listed on aeronautical charts since they do not cause a nonparticipating aircraft to change its flight path.

QUESTIONS?

- Put any questions in the chat. I will be sure to post.

OTHER AIRSPACE AREAS

OTHER AIRSPACE AREAS

LOCAL AIRPORT ADVISORY

- An advisory service provided by Flight Service facilities, located on the landing airport, using a discrete ground-to-air frequency or the tower frequency when the tower is closed.

- Services include local airport advisories, automated weather reporting with voice broadcasting, and a continuous Automated Surface Observing System (ASOS)/Automated Weather Observing Station (AWOS) data display, other continuous direct reading instruments, or manual observations available to the specialist.

MILITARY TRAINING ROUTES

- Military Training Routes (MTRs) are routes that military aircraft fly during training.

- Military aircraft can exceed 250-knots below 10,000 feet.

- MTRs with no segment above 1500' are identified by four-digit numbers; e.g., VR1007, etc. and flown under Visual Flight Rules.

- MTRs that include one or more segments above 1500' AGL are identified by three or fewer digit numbers; e.g., IR21, VR207  etc. and flown under Instrument Flight Rules.

- VFR routes include the letters VR and are not ATC-controlled

- IFR routes are under ATC control and include the letters IR.

MTR EXAMPLES

TEMPORARY FLIGHT RESTRICTIONS (TFRS)

- TFR Areas Relating to National Security are indicated with a broken blue line

  - Note - only TFRs relating to National Security are charted

- TFR is a type of Notice to Airmen (NOTAM) it includes:

  - Location of the temporary restriction

  - Time period

  - Area in statute miles, and altitudes affected

  - FAA coordination facility and telephone number

  - Reason for the restriction

  - The pilot should check the NOTAMs as part of flight planning.

TEMPORARY FLIGHT RESTRICTIONS

- TFR - area where air travel is restricted due to a hazardous condition, a special event, or a general warning for the entire airspace e.g. include:

  - Protect persons and property in the air or on the surface from an existing or imminent hazard

  - Provide a safe environment for the operation of disaster relief aircraft.

TEMPORARY FLIGHT RESTRICTIONS - CONTINUED

- Prevent an unsafe congestion of aircraft above an incident or event, e.g. Superbowl

- Protect declared national disasters for humanitarian reasons in the State of Hawaii.

- Protect the President, Vice President, or other public figures.

- Provide a safe environment for space agency operations.

EXAMPLE TFR

PARACHUTE JUMP AIRCRAFT OPERATIONS

- Operations are published in the Chart Supplement U.S.

- Sites that are used frequently are depicted on sectional charts.

PUBLISHED VFR ROUTES

- Are for transitioning around, under, or through some complex airspace.

- Terms such as VFR flyway, VFR corridor, Class B airspace VFR transition route, and terminal area VFR route have been applied to such routes.

- Found on VFR terminal area planning charts.  Example: by NYC

TERMINAL RADAR SERVICE AREAS TRSA

- Areas where participating pilots can receive additional radar services.

- The purpose of the service is to provide separation between all IFR operations and participating VFR aircraft.

- Depicted on sectional charts with a solid grey line.

NATIONAL SECURITY AREAS (NSAS)

- Anywhere there is a requirement for increased security and safety of ground facilities.

- Flight in NSAs may be temporarily prohibited by regulation under the provisions of 14 CFR part 99

- Prohibitions are disseminated via NOTAM.

- Pilots are requested to voluntarily avoid flying in the area.

AIR DEFENSE IDENTIFICATION ZONES (ADIZ)

- Surrounds the country’s perimeter protecting the country from unknown aircraft

- The government treats unidentified aircraft entering the ADIZ as possible threats possible interception by military aircraft.

- Sectional charts depict ADIZ boundaries with a magenta line with dots.

QUESTIONS?

- Put any questions in the chat. I will be sure to post.