AIRPORT

OPERATIONS

AIRPORT OPERATIONS

- Airport Categories as defined by law

  - Commercial Service Airports

    - Publicly owned & at least 2,500 passenger boardings per year includes international flights stopping for non-traffic purposes - refueling

  - Cargo Service Airports

    - Served by aircraft transporting only cargo greater than 100 million lbs.

AIRPORT OPERATIONS

- Reliever Airports

- FAA-designated to reduce congestion at commercial service airports.

- General Aviation Airports

- All remaining airports not in the above categories.

- Largest group in the U.S. airport system.

AIRPORTS

- Two types of airports—towered and non-towered

- Within those there are:

  - Civil Airports for the general public

  - Military/Federal Government

  - Private Airports - not open to the general public

- Facilities include seaplane bases, heliports, tilt rotor aircraft & can include an area used or intended for airport facilities.

Questions?

RUNWAYS & ORIENTATION

AIRPORT RUNWAYS

- It is essential you to know the meanings of the signs, markings, and lights that are used on airports as surface navigational aids.

- Navigational aids include:

  - The airport diagram or layout

  - Runways in use

  - The direction of departing and arriving aircraft

AIRPORT RUNWAYS

- The most dangerous part of flying an aircraft is generally considered to be the takeoff and landing phases

- Low altitude & speed of the aircraft leaves little margin or error

- Wind shear, crosswinds, fog, and low visibility are most critical near the ground.

  - Sudden changes in wind can destabilize the aircraft during landing or takeoff.

RUNWAY ORIENTATION

- Runways are aligned with the prevailing wind for better lift and control.

- The runway heading is expressed in magnetic degrees, rounded to the nearest 10° and shortened to two digits.

- Ex: A runway aligned to 273° magnetic - Runway 27

AIRPORT RUNWAY EXAMPLE

- Opposite ends of the same runway differ by 180°

- Example - Runway 27 on one end, Runway 9 on the other

RUNWAY ORIENTATION

- Runway numbers range from 01 to 36, representing compass headings (10° increments).

- Parallel runways use letters - L (Left), C (Center), R (Right).

- Ex: Two parallel runways heading 090° would be 09L and 09R.

Questions?

TRAFFIC PATTERNS

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

- Airports use a standard pattern that provides an organized flow for takeoffs and landings

- These patterns could change if they have a control tower, unique terrain features or obstacles

- Never assume aircraft will be following the standard left-hand pattern there could be a reason that the pilot deviates

- If in an area with manned aircraft your best option might be to land until the area is clear

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

- Even though propeller aircraft cruise much higher, they descend through lower altitudes near airports.

- Approach/Departure Paths: Aircraft may be at 500 –1,500’ AGL several miles from the airport.

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

- Common altitudes for propeller aircraft are 600’ to 1500’ AGL

- Patterns are normally left unless indicated by the ATC

- Right patterns indicated with a (RP)

- Left hand is not labeled on chart

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

Significant items of the traffic pattern:

- Enter pattern in level flight, abeam the midpoint of the runway, at pattern altitude.

  - 1,000' AGL is recommended altitude unless otherwise established

- Maintain pattern altitude until abeam approach end of the landing runway on downwind leg.

- Complete turn to final at least ¼ mile from the runway.

TRAFFIC PATTERNS FOR MANNED AIRCRAFT

Significant items in traffic pattern continued:

- After takeoff or go-around, continue straight ahead until beyond departure end of runway.

- If remaining in the traffic pattern, initiate a turn to crosswind leg beyond the departure end of the runway within 300’ of pattern altitude.

- If departing the traffic pattern, continue straight out, or exit with a 45° turn beyond the departure end of the runway, after reaching pattern altitude.

Questions?

AIRPORT SIGNS & MARKINGS

AIRPORT SIGNS

- Location sign - Black with yellow letters – Taxiway or runway location

- Mandatory Instruction Sign -Red sign with white letters - Indicate areas where pilots must stop unless cleared by the ATC

- Found on runways, critical areas and restricted zones.

AIRPORT SIGNS

- Direction and Destination Signs: yellow backgrounds, black writing, and arrows, used to locate various areas.

- ILS Critical Area Holding Position - Hold when ILS critical area in use (usually during low visibility).

22

AIRPORT SIGNS

- Two solid lines and two broken lines stretching the width of the taxiway or runway.

- When approaching taxiway holding lines from the side with the continuous line, the pilot should not cross the lines without ATC clearance.

AIRPORT SIGNS

- Located next to the Runway Holding Position Markings on taxiways or runway intersections. Runway 15 left and runway 33 is right.

- Aircraft should not proceed beyond this sign until cleared by Air Traffic Control (ATC).

Questions?

RUNWAY QUESTIONS

EXAMPLE RUNWAY QUESTION

- You are monitoring the CTAF Frequency at a local airport and the pilot says he is left downwind of runway 16.  What is the heading of the aircraft?

- 160° + 180° = 340° or North West

EXAMPLE RUNWAY QUESTION

While monitoring the CTAF you hear the pilot announce that they are midfield left downwind to RWY 13.  Where would the aircraft be relative to the RWY?

  - The aircraft is East

  - The Aircraft is South

  - The Aircraft is West

EXAMPLE RUNWAY QUESTION

While monitoring the CTAF you hear the pilot announce that they are midfield left downwind to RWY 4.  Where would the aircraft be relative to the RWY?

  - The aircraft is East

  - The Aircraft is South

  - The Aircraft is West

EXAMPLE RUNWAY QUESTION

While monitoring the CTAF you hear the pilot announce that they are midfield left downwind to RWY 12.  What is the heading of the aircraft ?

  - 120 degrees

  - 180 degrees

  - 300 degrees

EXAMPLE RUNWAY QUESTION

While monitoring the CTAF you hear the pilot announce that they are midfield left downwind to RWY 24.  What is the heading of the aircraft ?

  - 60 degrees

  - 50 degrees

  - 70 degrees

EXAMPLE RUNWAY QUESTION

While monitoring the CTAF you hear the pilot announce that they are on the short final of RWY 18.  Where would the aircraft be relative to the airport?

  - The aircraft is North

  - The Aircraft is South

  - The Aircraft is West

QUESTIONS OR  COMMENTS

AIRPORT SECURITY & WILDLIFE HAZARDS

AIRPORT SECURITY

- If drone operation requires entry onto the airport property you must determine if that area is a Security Identification Display Area (SIDA)

- Have Airport-Issued Identification badge visible at all times and follow procedures in CFR 49 Part 1542

- Without a badge being displayed movement in these areas are prohibited

WILDLIFE AIRCRAFT STRIKES

WILDLIFE AIRCRAFT STRIKES

- If your drone collides with wildlife, you are not required to report it, but you can report to the FAA Wildlife Strike Database

- Do not harass wildlife - violates FAA and environmental laws

- If wildlife is observed near your operation, land immediately

QUESTIONS OR  COMMENTS

Please post and it will be answered