WEATHER EFFECTS ON AIRCRAFT PERFORMANCE PART 1

WEATHER EFFECTS ON PERFORMANCE

- Pilots must understand how weather affects drone

- Every drone has weather limits set by the manufacturer

  - Exceeding those limits increases the chance of:

    - Loss of control

    - Crashes

    - Injury or property damage

    - If weather exceeds aircraft performance limits, don’t fly

EFFECTS ON PERFORMANCE

- Pilots can not control the weather  - only their decisions

- Pilots care about weather because it affects air density which affects the drone performance

  - Never accept weather related risks unless the mission truly justifies it.  For example:  If the weather is bad…

    - Inspecting a roof 🡪 delay the flight

    - Searching for a child whose life maybe in danger 🡪 higher risk may be justified

    - Most operations do NOT justify high weather risk

WEATHER

- The current condition of the atmosphere at a specific time and place

- Includes: temperature, wind, pressure moisture (humidity, precipitation)

- Weather happens because the sun heats the earth unevenly

- Uneven heating causes air movement

- Moving air = weather

AIR PRESSURE

- Air pressure is the weight of the air above you caused by gravity pulling air downward

  - More air above you 🡪 higher pressure

  - Less air above you 🡪 lower pressure

  - As altitude increases:

    - Air pressure decreases

    - Air density decreases

MEASUREMENT OF ATMOSPHERE PRESSURE

- International Standard Atmosphere - provides a common reference for aircraft performance data.

- Pressure = weight of air pushing down

MEASUREMENT OF ATMOSPHERE PRESSURE

- Atmospheric pressure can be reported as inches of Mercury (inHg) or Milibars (mb)

- Standard sea level conditions:

  - Pressure: 29.92 inHg or 1,013.2 mb

  - Temperature: 59° F (15° C)

- Test tip ** FAA tests both units

AIR DENSITY

- Air density is the amount of air molecules in a given volume of air.

- Why it matters:

  - Air density changes depending on your altitude, which directly affects how a drone flies.

  - Because drones rely on air molecules for lift, thrust, and stability, their performance can shift significantly during a single mission as they move through different atmospheric layers.

HIGH VS. LOW AIR DENSITY

- High Air Density (Thick Air):

  - More air molecules

  - Better lift and propeller efficiency

  - Better aircraft performance

- Low Air Density (Thin Air):

  - Fewer air molecules

  - Reduced lift and thrust

  - Worse aircraft performance

AIR DENSITY – (VERY TESTABLE)

- High, Hot, and Humid = Low Air Density

- High Altitude - Air pressure decreases with altitude

  - Fewer air molecules → reduced lift

- High Temperature - Warm air expands

  - Molecules spread out → lower density

- High Humidity - Water vapor displaces oxygen and nitrogen

  - Moist air is actually less dense than dry air

- Exam tip: If a question mentions mountains, humid conditions or summer think LOW air density.

DENSITY ALTITUDE

- Density altitude is pressure altitude corrected for nonstandard temperature.

- As temperature and altitude increase air density and aircraft performance decreases

- Density altitude describes how thick or thin the air feels to the aircraft

- Density Altitude  is like the “feeling” altitude of the airplane.

- It describes how high the aircraft feels like it’s flying, based on how thick or thin the air is.

- Density Altitude:  Density Altitude has a direct effect on the aircraft performance

- Relationship Between Air Density and Aircraft Performance

  - Lower Density Altitude (High Air Density /thick air) – Increased aircraft performance.

  - Higher Density Altitude (Low Air Density /thin air) - Reduced aircraft performance.

- Key Concept:

  - A decrease in air density means a high-density altitude.

  - An increase in air density means a lower-density altitude.

- Pressure altitude - altitude when an altimeter is set to 29.92 Hg

- Standard temperature is 15° C at sea level only.

- Temperature decreases approximately:

  - 2 °C (or 3.5 °F) per 1,000 feet of altitude above sea level.

- If temperature is higher than standard, density altitude increases and performance decreases

- Density Altitude The altitude in the standard atmosphere that corresponds to a specific air density.

- Air Density – a measurement of the amount of air molecules in a given volume.

- Test tip - Density altitude is not where you are – it’s how the air behaves.

DENSITY ALTITUDE - MUST KNOW

- High density altitude = poor UAS performance

- Expect:

  - Reduced lift

  - Reduced climb rate

  - Reduced propeller efficiency

  - Shorter battery endurance

***** High, hot, and humid conditions increase density altitude and decrease drone performance ****

AIR DENSITY

- Summary of conditions and how they affect air density

PRESSURE

- Atmospheric Pressure

  - More collisions = higher pressure

  - Less collisions = lower pressure

- Two ways to change pressure

  - Change the amount of air molecules or

  - Change the speed of the molecules

PRESSURE AND WEATHER

EFFECTS OF PRESSURE ON DENSITY

- Since air is a gas, it is considered a compressible fluid

- You can “push” on a volume of air to decrease its volume causing an increase in pressure

- Compressible fluids are affected by changes in pressure

EFFECTS OF PRESSURE ON DENSITY

- In the atmosphere, both temperature & pressure decrease with altitude

- Usually, an increase in altitude causes a more rapid drop in pressure than temperature

  - So, pressure changes are usually the more dominant factor

  - Hence, pilots can expect the density to decrease with altitude

HUMIDITY

- Is the amount of water vapor in the air

- Expressed as a percent (%) concentration of the maximum amount possible at any given temperature

- Warm air can hold more water vapor

- Colder air can hold less

- H2O weighs less than N2 and O2

- More water = lighter air = less dense

HUMIDITY

- Perfectly dry air - contains no water vapor

  - Relative humidity - 0%

- Saturated air - cannot hold any more water vapor

  - Relative humidity -100%

- 50% Humidity - The air has 50% of the maximum H2O the air can hold at that temperature

- Both temperature & pressure affect the humidity

HUMIDITY

- Water vapor weighs less than air

  - Water vapor (H2O) weighs ~ 0.80 g/l

  - Oxygen (O2) weighs ~ 1.43 g/l

  - Nitrogen (N2) ~ 1.25 g/l

- The more water in the air the lighter air is

- The higher the humidity, the higher the density altitude

HUMIDITY

Recall:

- Density has great effect on aircraft performance

- Higher humidity is lower air density

- Lower density = decreased performance

- Lower pressure is LESS lift

- Lower density is less air “pushed” down

QUESTIONS

Any questions be sure to leave a comment

WEATHER EFFECTS ON PERFORMANCE - PART 2

WIND

WIND

- Obstacles cause turbulence

- The amount of turbulence depends on the size of the obstacle and the velocity of the wind flowing over or around it

- May experience turbulence around man-made structures like hangars, natural obstructions, tree lines, mountains, bluffs, or canyons

- Can cause issues with video & photos

WIND

- Exercise extra caution when flying near open fields and water

- The lack of obstacles can allow the wind to increase speed relative to areas with obstacles

- Wind speeds & gusts could exceed the manufacturing limits of the drone.

WIND

WIND

- As air flows down the leeward side of a mountain the air  follows the contour of the terrain & is increasingly turbulent

- On the leeward side winds will tend to push an aircraft into the side of the mountain

- The stronger the wind, the greater the downward pressure and turbulence

WIND

- As air flows around/through buildings turbulence increases

- The wind will try to make a sharp turn around a corner or peak but cannot and causes the turbulence on the leeward side

LOW LEVEL WIND SHEAR

- Wind shear - sudden change in wind speed and/or direction over a short distance

- Can occur at any altitude causing updrafts and downdrafts as well as horizontal movement.

- Especially dangerous when the aircraft is closer to the ground.

- Associated with passing frontal systems, thunderstorms, temperature inversions, and strong upper-level winds > 25 KT

WIND

QUESTIONS

Any questions be sure to leave a comment

ATMOSPHERIC STABILITY

ATMOSPHERIC STABILITY

- Convective Currents: Convective Currents are areas where uneven heating of the air causes air to move up or down and create bumpy, turbulent flights.

- Warm air rises up (updrafts), and cool air sinks down (downdrafts).

ATMOSPHERIC STABILITY

- How stable the atmosphere depends on its ability to resist vertical motion.

- When the atmosphere is unstable, vertical air motions can grow stronger, producing turbulent airflow and convection

- Instability can lead to significant turbulence, extensive vertical clouds, and severe weather

ATMOSPHERIC STABILITY

- The combination of moisture and temperature determine the stability of the air and the weather.

- When one type of air mass overlies another conditions will change with height

- Cool, dry air is very stable and resists vertical movement

  - Good and generally clear weather.

ATMOSPHERIC STABILITY

- If air is moist and warm (summer tropical) - instability occurs (vertical movement)

  - Thunderstorms can happen daily due to the instability of the air.

  - Stratiform clouds form horizontally compared to cumuliform clouds that form vertically.

ATMOSPHERIC STABILITY

QUESTIONS

Any questions be sure to leave a comment

TEMPERATURE INVERSIONS

TEMPERATURE INVERSIONS

- Normally as air rises and expands in the atmosphere, the temperature decreases.

- When the temperature of the air rises with altitude, a temperature inversion exists.

- Inversion layers are commonly shallow layers of smooth, stable air close to the ground.

TEMPERATURE INVERSIONS

- The temperature of the air increases with altitude which is the top of the inversion.

- The air at the top of the layer acts as a lid, keeping weather and pollutants trapped below

- Surface-based temperature inversions occur on clear, cool nights when the air close to the ground is cooled by the lowering temperature of the ground

TEMPERATURE INVERSIONS

- Low-Level Temperature Inversion - when a layer of warmer air sits above cooler air near the ground.

  - Unusual because normally, air gets cooler as you go higher.

  - Can make the air feel stable, but it can also trap pollution and create fog or smog close to the ground.

TEMPERATURE INVERSIONS

- Frontal inversions occur when warm air spreads over a layer of cooler air, or cooler air is forced under a layer of warmer air.

  - Can cause unstable air and worsen flight conditions

TEMPERATURE INVERSIONS

- If the relative humidity of the air is high, it can contribute to the formation of clouds, fog, haze, or smoke resulting in diminished visibility in the inversion layer.

TEMPERATURE AND DEW POINT

- The relationship between dew point and temperature defines the concept of relative humidity.

- Dew point is the temperature at which the air can hold no more moisture.

- When air cools to its dew point, it becomes fully saturated, causing water vapor to condense and form fog, dew, frost, clouds, rain, or snow.

TEMPERATURE AND DEW POINT

- When temperatures are above freezing, condensed moisture called dew or frost settles on grass, buildings, cars, and aircraft surfaces

- This can lower performance and lift affecting the ability to take off  because it adds weight and interrupts the flow of air over the wings and rotors

QUESTIONS

Any questions be sure to leave a comment

WEATHER EFFECTS ON PERFORMANCE - PART 3

CLOUDS

CLOUDS

- A cumuliform cloud is a puffy, white cloud with a flat base - looks like cotton balls.

- Often form on sunny days and can grow tall and large, sometimes leading to thunderstorms.

- The thunderstorms can produce large hailstones, damaging lightning, tornadoes, and a lot of precipitation

CLOUDS

- Cumulonimbus clouds are one of the most dangerous cloud types to drone operators.

  - Occur individually or in groups

- Since rising air currents cause cumulonimbus clouds, they are extremely turbulent and pose a significant hazard to flight safety.

CLOUDS

- Rising air currents cause cumulonimbus clouds to form

- Warm air in atmosphere causes convection, which is the way air rises upwards because it is less dense than surrounding air

  - Extremely turbulent

- A small UA could experience updrafts and downdrafts exceeding 3,000 fpm.

CLOUDS

- Standing lenticular altocumulus clouds - dangerous to drone operators

- Standing lenticular altocumulus clouds are formed on the crests of waves created by barriers in the wind flow i.e.; mountains

  - The wind can be very strong blowing through such clouds but can show little movement.

- Characterized by smooth, polished edges.

- Strong turbulence and should be avoided.

FRONTS

FRONTS

- As air masses move out of their regions, they come in contact with other air masses of different properties.

- The zone between two different air masses is a frontal zone or front.

  - Across this area, temperature, humidity & wind can change rapidly over short distances.

FRONTS

- Warm front -  when a warm mass of air advances and replaces a body of colder air.

- As the warm front approaches, the barometric pressure continues to fall until front passes.

- Warm fronts – advanced warning take days to pass trough

FRONTS

- A cold front occurs when a mass of cold, dense, and stable air advances and replaces a body of warmer air

- A high dew point and falling barometric pressure mean a cold front is coming.

- Cold fronts – fast approaching with little warning

MOUNTAINS

MOUNTAIN FLYING

- When planning a flight over mountainous terrain, gather as much preflight information on the area as possible including:

  - Cloud reports

  - Wind direction

  - Wind speed

  - Stability of air

- Adequate information may not always be available, so stay alert of the environment.

MOUNTAIN FLYING

- Wind at mountain top level versus halfway up the mountain can greatly differ.

  - Exercise caution

- Stratified clouds mean stable air.

- Standing lenticular or rotor clouds - expect turbulence on the lee of mountains but relative smooth flight on the windward side.

- Convective clouds on the windward side of mountains mean unstable air expect turbulence close to the mountain on either side.

QUESTIONS

Any questions be sure to leave a comment

STRUCTURAL ICING

STRUCTURAL ICING

- Structural icing - Ice forms on aircraft’s wings, body and propellers

  - Very dangerous

- Can affect the ability to produce lift, increases drag and increase weight

  - Many flights get canceled until the plane is deiced.

STRUCTURAL ICING

- Two conditions are necessary for structural icing in flight:

  - 1. Must be flying through visible water such as rain or cloud droplets

  - 2. The temperature at the point where the moisture strikes the aircraft must be 0° C or colder.

- Aerodynamic cooling can lower temperature of an airfoil to 0° C even though the ambient temperature is a few degrees warmer.

THUNDERSTORMS

THUNDERSTORMS

- Thunderstorms go through three stages:

  - Cumulus Stage

  - Mature Stage

  - Dissipating Stage

- Transitions are subtle and hard to see visually.

- A thunderstorm may contain multiple cells in different stages simultaneously.

THUNDERSTORM LIFE CYCLE

CUMULUS STAGE

- Key Characteristics:

  - Every thunderstorm begins as a cumulus cloud.

  - Dominated by a strong updraft from surface to cloud top.

  - Cloud growth may exceed 3,000 ft/min.

CUMULUS STAGE

- Rapidly building cumulus clouds → hazardous for small UA (risk of turbulence and strong vertical air movement).

- Water droplets grow as the cloud develops.

- Updraft carries liquid water above freezing level → icing hazard.

- As droplets grow heavier and begin to fall:

  - Cold rain drags air downward.

  - Updraft + new downdraft coexist → transition to Mature Stage.

MATURE STAGE

- Precipitation starts falling → begins mature stage → presence of a strong downdraft

- Downdraft Behavior - Cold rain cools the air, preventing heating.

- Downdraft remains colder than surrounding air → accelerates downward.

- Downdraft speeds may exceed 2,500 ft/min.

MATURE STAGE

- Down rushing air spreads outward on the ground producing:

  - Strong, gusty surface winds

  - Sharp temperature drop

  - Rapid pressure rise

- Leading edge of surface wind surge:

  - Plow wind

  - First wave: “first gust”

MATURE STAGE

- Updrafts reach max strength

- Strong updrafts AND downdrafts → severe vertical wind shear.

- Mature stage is the most dangerous for:

  - Turbulence

  - Wind shear

  - Hail

  - Lightning

  - Microbursts and strong downdrafts

  - Highest risk period for aviation and UAS operations.

DISSIPATING STAGE

- Dominated entirely by downdrafts.

- Rain ends as the storm weakens.

- Downdrafts diminish → storm rapidly falls apart

- When downdrafts cease, the cell has fully dissipated.

- Remaining clouds are generally harmless.

CEILING

- The lowest layer of clouds reported as being broken or overcast, or the vertical visibility into an obscuration like fog or haze.

- Clouds are reported as broken when five-eighths to seven-eighths of the sky is covered.

- Overcast - entire sky is covered with clouds

- Ceiling information is reported by the aviation routine weather report (METAR) and automated weather stations

- Don’t forget you need to stay 500 ft below the clouds

VISIBILITY

- The greatest horizontal distance at which prominent objects can be seen with the naked eye.

- METARs and other automated weather systems include current visibility as part of standard weather reporting

- Meteorologists provide predicted visibility during official weather briefings.

- Visibility requirement for sUAS operations – 3SM

QUESTIONS

Any questions be sure to leave a comment