- Which of the following is NOT covered under Part 107?
A) Civil small UAS operations
B) Public aircraft operations
C) Airman certification
D) Registration of small UAS

- Who must hold a Remote Pilot Certificate with a sUAS rating?
A) Visual Observer
B) RPIC
C) Ground Crew
D) Payload Operator

- What is the RPIC responsible for during flight?
A) Only launching the drone
B) Final authority on sUAS operation
C) Monitoring weather
D) Communicating with ATC

- Which role is optional but recommended under Part 107?
A) RPIC
B) VO
C) Ground Crew
D) Payload Operator

- What should the RPIC do if visual contact with the drone is lost for more than 15 seconds?
A) Continue flying
B) Land immediately
C) Initiate Failsafe Return-to-Home
D) Contact FAA

- How many UAS can a person operate at one time under Part 107?
A) Unlimited
B) Two
C) One
D) Three

- What is the minimum visibility required from the control station?
A) 1 statute mile
B) 2 statute miles
C) 3 statute miles
D) 5 statute miles

- What is the maximum allowable altitude for sUAS flight under Part 107?
A) 200 feet AGL
B) 300 feet AGL
C) 400 feet AGL
D) 500 feet AGL
Answer: C) 400 feet AGL

- What is the minimum distance a drone must maintain from clouds?
A) 100 feet below
B) 500 feet below and 2000 feet horizontally
C) 1000 feet above
D) 300 feet horizontally

- What must be reported to the FAA within 10 calendar days?
A) Any flight
B) Any battery failure
C) Serious injury or property damage over $500
D) Weather delays

- What is the maximum allowable groundspeed for sUAS?
A) 87 knots
B) 100 knots
C) 120 knots
D) 150 knots

- How far in advance should waiver applications be submitted?
A) 15 days
B) 30 days
C) 60 days
D) 90 days

- Which of the following operations can be waived?
A) Flying over people
B) Flying from a moving aircraft
C) Flying without registration
D) Flying without Remote ID

- What is Remote ID often compared to?
A) License plate
B) GPS tracker
C) Flight recorder
D) Radar

- Which category requires an airworthiness certificate under Part 21?
A) Category 1
B) Category 2
C) Category 3
D) Category 4

- Which category may operate over people only in restricted-access sites?
A) Category 1
B) Category 2
C) Category 3
D) Category 4

- What lighting is required for night operations?
A) Flashlight
B) Anti-collision lights visible for 3 statute miles
C) Red beacon
D) Infrared lights

- Which of the following operations is explicitly excluded from Part 107 applicability?
A) Commercial sUAS operations within U.S. borders
B) Model aircraft flown for recreation under Part 101
C) Public safety drone operations under Part 107
D) Civil sUAS operations under 55 lbs

- Under Part 107, who holds final authority over the operation of the sUAS?
A) Visual Observer
B) Ground Crew
C) Remote Pilot in Command
D) Person manipulating the flight controls

- Which of the following is a required action for the RPIC before flight?
A) Submit a flight plan to ATC
B) Conduct a preflight inspection of aircraft and control station systems
C) Notify local law enforcement
D) Obtain a waiver for VLOS operations

- What is the minimum requirement for someone manipulating the flight controls of an sUAS?
A) Must be a certified RPIC
B) Must be under direct supervision of an RPIC
C) Must be a VO
D) Must have a waiver from the FAA

- Which of the following statements about Visual Observers (VOs) is correct?
A) VOs are mandatory for all Part 107 operations
B) VOs must hold a Remote Pilot Certificate
C) VOs are optional but recommended depending on flight complexity
D) VOs must be located at the control station

- Which of the following is a requirement for autonomous operations under Part 107?
A) RPIC must maintain manual control at all times
B) RPIC must be able to command the aircraft to land immediately
C) RPIC must file a flight plan with ATC
D) RPIC must use ADS-B Out

- Which regulation governs the registration of small unmanned aircraft?
A) Part 91
B) Part 107
C) Part 47 or Part 48
D) Part 61

- What must be made available upon request by law enforcement during an sUAS operation?
A) Flight logs only
B) Remote Pilot Certificate and identification
C) FAA waiver documentation only
D) Insurance documents

- Which of the following conditions requires accident reporting to the FAA within 10 days?
A) Any drone crash
B) Damage exceeding $500 in repair costs
C) Battery failure
D) Loss of GPS signal

- Which of the following is considered a physical or mental incapacitation under Part 107?
A) Wearing sunglasses
B) Use of antihistamines causing drowsiness
C) Operating in cold weather
D) Flying with corrective lenses

- Which of the following is prohibited without prior authorization?
A) Flying in Class G airspace
B) Flying in Class B airspace
C) Flying below 100 feet AGL
D) Flying over private property

- What is the RPIC’s responsibility regarding other aircraft?
A) Maintain radio contact
B) Yield right of way and avoid collision
C) Fly above manned aircraft
D) Use ADS-B Out

- What is the purpose of Remote ID?
A) To enable autonomous flight
B) To allow drone delivery
C) To identify UA in the NAS
D) To track battery performance

- Which of the following is NOT a method to comply with Remote ID requirements?
A) Standard Remote ID UA
B) Remote ID Broadcast Module
C) FAA-Recognized Identification Area
D) ADS-B Out

- What must a Remote ID UA do before takeoff?
A) Broadcast a GPS signal
B) Pass a self-test
C) Connect to ATC
D) Upload flight plan

- Which of the following is true about FAA-Recognized Identification Areas (FRIAs)?
A) Only commercial operators may apply
B) They allow flight without Remote ID
C) They are valid for 12 months
D) They require ADS-B Out

- Which category of UA requires an airworthiness certificate under Part 21?
A) Category 1
B) Category 2
C) Category 3
D) Category 4

- Which category of UA may operate over people without a waiver if it weighs less than 0.55 lbs and has no exposed rotating parts?
A) Category 1
B) Category 2
C) Category 3
D) Category 4

- Under Part 107, which of the following is permitted when transporting property for compensation or hire?
A) Flying across state lines
B) Operating from a moving aircraft
C) Conducting operations wholly within a single state
D) Flying over open-air assemblies

- Which of the following is prohibited during sUAS operations under Part 107?
A) Operating from a moving land vehicle in a sparsely populated area
B) Operating from a moving aircraft
C) Operating from a stationary watercraft
D) Operating from a moving vehicle with a VO present

- Which of the following would be considered a careless or reckless operation under Part 107?
A) Flying with a VO in Class G airspace
B) Dropping objects from a UA without hazard mitigation
C) Flying below 100 feet AGL in rural areas
D) Operating with anti-collision lights during daylight

- Which of the following substances or conditions would disqualify a person from participating in sUAS operations under Part 107?
A) Use of caffeine
B) Consumption of alcohol within the past 6 hours
C) Blood alcohol concentration of 0.04%
D) Use of prescription glasses

- What must occur if a Remote ID Broadcast Module fails its self-test prior to flight?
A) The RPIC may override the failure
B) The UA may fly only in Class G airspace
C) The UA must not be operated
D) The RPIC must notify the FAA

- Which of the following is a requirement for Category 2 UA operations over people?
A) UA must weigh less than 0.55 lbs
B) UA must have exposed rotating parts
C) UA must not exceed 11 foot-pounds of kinetic energy upon impact
D) UA must be operated only at night

- Which of the following is true for Category 3 UA operations over people?
A) UA may maintain sustained flight over open-air assemblies
B) UA must be operated only in uncontrolled airspace
C) UA may operate over people only in restricted-access sites with notice
D) UA must be equipped with ADS-B Out

- Which of the following is a recommended mitigation strategy for night illusions during sUAS operations?
A) Use of infrared goggles
B) Looking directly at bright lights
C) Scanning off-center by 5–10 degrees
D) Flying above 400 feet AGL